# Sensor-Fusion Water Stress Detection with Self-Calibrating Edge AI

We propose a hybrid system that fuses free Sentinel-2 multispectral imagery (10-20 m, 5-day revisit) with a sparse network of low-cost soil-moisture and canopy-temperature sensors (one node per ~5 farms).

Core idea: the ground nodes continuously self-calibrate the satellite model using a lightweight Gaussian-process residual correction, so no expert calibration is needed per farm. This addresses the scaling constraint directly.

Water stress is predicted from a physically-grounded Crop Water Stress Index (CWSI) augmented by an ML model trained on canopy temperature and NDRE/NDWI indices. We target 3-5 day lead time before visible wilting, validated against a held-out season of ground truth.

Edge nodes run on solar + battery and cache alerts locally, syncing over intermittent connectivity via LoRaWAN to a gateway. Alerts are delivered as simple SMS with an irrigation recommendation.

Validation plan: 2 growing seasons, 30 plots, compare predicted stress onset to pressure-chamber leaf water potential; target >85% precision at 4-day lead time. Cost: ~$40 per farm amortized (shared nodes).

Risks: LoRa range in hilly terrain, sensor drift; mitigated by redundant nodes and periodic self-calibration.